Analysis of multi-story multi-bay RC plane frame

Input data

Number of stories - nst=5 ,  Number of bays - nb=3

Story height - hst=2.85 m,     Bay length- lb=4 m

Slab thickness - hpl=18 cm

Cross section of columns -  bc=25 cm ,  hc=60 cm

Cross section of beams -     bb=25 cm ,  hb=40 cm

Joint coordinates -  nJ=24

xJ=0 4 8 12 0 4 8 12 0 4  ... 12 m, yJ=0 0 0 0 2.85 2.85 2.85 2.85 5.7 5.7 ... 14.25 m

Elements - [J1; J2] -  nE=35

transpeJ=12345678910⋯23567891011121314⋯24

Element endpoint coordinates

x1e=xJ.eJe,1 , y1e=yJ.eJe,1 ,  x2e=xJ.eJe,2 , y2e=yJ.eJe,2

Element length -  le=x2e-x1e2+y2e-y1e2

Element direction - ce=x2e-x1ele , se=y2e-y1ele

Transformation matrix

Diagonal 3x3 block - te=hpce;se;0 | -se;ce;0 | 0;0;1

Generation of the full transformation matrix

Te=addte;addte;matrix6;6;1;1;4;4

Supports -  nc=4

c=11020 kN/ m1020 kN/ m0 kNm21020 kN/ m1020 kN/ m0 kNm31020 kN/ m1020 kN/ m0 kNm41020 kN/ m1020 kN/ m0 kNm

Unit weights of building materials

- concrete - γc=25 kN/ m3			     - screed - γscr=21 kN/ m3

- finishes - γfin=18 kN/ m3			     - brickwork - γbw=16 kN/ m3

- plaster/render - γpla=16 kN/ m3		     - insulation - γins=0.5 kN/ m3

Loads

Total halfwidth of adjacent plate spans - a=5 m2=2.5 m

Self weight

Plate - gpl=hpl·a·γc=11.25 kN/ m

Beam - gb=bb·hb-hpl·γc=1.38 kN/ m

Total for beam - sw=gpl+gb=12.62 kN/ m

Column - gc=bc·hc·γc=3.75 kN/ m

Dead loads

Screed - gscr=8 cm·a·γscr=4.2 kN/ m

Finishes - gfin=2 cm·a·γfin=0.9 kN/ m

Plaster ceiling - gpls=2 cm·a·γpla=0.8 kN/ m

Brick wall - gbw=25 cm·hst-hb·γbw=9.8 kN/ m

Wall insulation - gins=15 cm·hst·γins=0.214 kN/ m

Wall plaster/render - gplw=2·2 cm·hst·γpla=1.82 kN/ m

Total dead load

dl=gscr+gfin+gpls+gbw+gins+gplw=17.74 kN/ m

Live load - ll=a·2 kN m2=5 kN/ m

Total load

On beams - pb=sw+dl·1.35+ll·1.5=48.49 kN/ m

On columns - pc=gc·1.35=5.06 kN/ m

Load values on elements

qx=0 0 0 0 0 0 0 0 0 0 ... 0 kN/ m

qy=-5.06 -5.06 -5.06 -5.06 -5.06 -5.06 -5.06 -5.06 -5.06 -5.06 ... -48.49 kN/ m

Scheme of the structure

Materials

Modules of elasticity - E=hp35 GPa=35 GPa

Poisson coefficients - ν=hp0.2=0.2

Shear modules - G=E2·1+ν=14.58 GPa

Assignments on elements

eM=fillvectorhpnE;1=1 1 1 1 1 1 1 1 1 1 ... 1

Cross sections

Calculation of effective flange width

l0=0.85·lb=3.4 m

beff=bb+min0.2·a+0.1·l0;0.2·l0=93 cm

Section 1 - b1=bc=25 cm , h1=hc=60 cm - columns

Section 2 - b2=bb=25 cm , h2=hb=40 cm  - beams

bf.=beff=93 cm , hf.=hpl=18 cm

Cross section properties

Area

Web - Aw=b⊙h=1500 1000 cm2

Flange - Af=bf-b⊙hf=0 1224 cm2

Total - A=Aw+Af=1500 2224 cm2

First moment of area - S=Aw⊙h2+Af⊙h-hf2=45000 57944 cm3

Geometrical center - zc=SA=300 260.54 mm

Second moment of area

Web - Iw=Aw⊙h⊙212+zc-h2⊙2=450000 169984 cm4

Flange - If=Af⊙hf⊙212+h-zc-hf2⊙2=0 62991.1 cm4

Total - I=Iw+If=450000 232975 cm4

Shear area - As=A1.2=1250 1853.33 cm2

Assignment on elements

Columns - eSC=fillvectorhpnCE;1=1 1 1 1 1 1 1 1 1 1 ... 1

Beams - eSB=fillvectorhpnBE;2=2 2 2 2 2 2 2 2 2 2 ... 2

All- eS=hpeSC;eSB=1 1 1 1 1 1 1 1 1 1 ... 2

Element stiffness matrix

Elastic properties for element "e"

EAe=EeM.e·AeS.e ,  EIe=EeM.e·IeS.e ,  GAse=GeM.e·As.eS.e

kse=12·EIeGAse·le2 ,  αe=EAele , βe=EIele3·1+kse

Stiffness matrix coefficients for element "e"

k11e=αe·mkN , k22e=12·βe·mkN , k23e=6·βe·le·1kN

k33e=4+kse·βe·le2·1kNm , k36e=2-kse·βe·le2·1kNm

Assembling the 3x3 stiffness matrix blocks for element "e"

kiie=hpk11e | 0;k22e;k23e | 0;k23e;k33e

kije=hp-k11e | 0;-k22e;k23e | 0;-k23e;k36e

kjie=transpkije

kjje=hpk11e | 0;k22e;-k23e | 0;-k23e;k33e

Full element stiffness matrix

kEe=stackaugmentkiie;kije;augmentkjie;kjje

Stiffness matrices obtained in local coordinates

kE1=184210500-184210500072402.71031740-72402.710317401031742022860-10317491759.5-1842105001842105000-72402.7-103174072402.7-103174010317491759.50-103174202286

nb1=nCE+1=21

kEnb1=194600000-194600000014950.729901.40-14950.729901.4029901.4801880-29901.439417.4-1946000001946000000-14950.7-29901.4014950.7-29901.4029901.439417.40-29901.480188

Stiffness matrices obtained in global coordinates

transpT1·kE1·T1=72402.70-103174-72402.70-1031740184210500-18421050-1031740202286103174091759.5-72402.7010317472402.701031740-18421050018421050-103174091759.51031740202286

transpTnb1·kEnb1·Tnb1=194600000-194600000014950.729901.40-14950.729901.4029901.4801880-29901.439417.4-1946000001946000000-14950.7-29901.4014950.7-29901.4029901.439417.40-29901.480188

Global stiffness matrix

K=10200-1031740000000⋯00102000000000⋯0-10317402022860000000⋯000010200-1031740000⋯00000102000000⋯0000-10317402022860000⋯000000010200-1031740⋯00000000102000⋯0000000-10317402022860⋯00000000001020⋯0⋮⋮⋮⋮⋮⋮⋮⋮⋮⋮⋱⋮0000000000⋯282474

Element load vector

Lateral load in local CS - qEe=-qx.e·se+qy.e·ce

Axial load in local CS - nEe=qx.e·ce+qy.e·se

Equivalent loads at element endpoints

FExe=qx.e·le2·1kN , FEye=qy.e·le2·1kN  , MEe=qEe·le212·1kNm

Load vector - FEe=hpFExe;FEye;MEe;FExe;FEye;-MEe

Global load vector

F=0 -7.21 0 0 -7.21 0 0 -7.21 0 0 ... 64.65

Results

Solution of the system of equations by PCG method

Z=slsolveK;F=0 -5.72×10-18 7.06×10-5 0 -1.03×10-17 3.03×10-6 0 -1.03×10-17 -3.03×10-6 0 ... 0.000283

Joint displacements

zJj=sliceZ;3·j-2;3·j		zj=roundzJjδz·δz·1000·mm;mm;1

Support reactions

ri=rowc;i , ji=take1;ri

Ri=-zJji·m;m;1·lastri;3

Joint J1 - 8.19 kN 571.77 kN 0 kNm 		Joint J2 - 0.174 kN 1027.2 kN 0 kNm

Joint J3 - -0.174 kN 1027.2 kN 0 kNm 		Joint J4 - -8.19 kN 571.77 kN 0 kNm

Element end forces

zEe=hpzJeJe,1;zJeJe,2

REe=colkEe·Te·zEe-Te·FEe;1·1;1;m;1;1;m·kN

Element internal forces

Ne;x=-take1;REe-nEe·x ,  Qe;x=take2;REe+qEe·x

Me;x=-take3;REe+take2;REe·x+qEe·x22

Axial forces diagram, kN

Shear forces diagram, kN

Bending moments diagram, kNm

Deformed shape

Shape function in relative coordinates ξ = x/l (with account to shear deflections)

Φ1e;ξ=11+kse·1+kse-kse·ξ-3·ξ2+2·ξ3

Φ2e;ξ=ξ·le·m-11+kse·1+kse2-2+kse2·ξ+ξ2

Φ3e;ξ=ξ1+kse·kse+3·ξ-2·ξ2

Φ4e;ξ=ξ·le·m-11+kse·-kse2-1-kse2·ξ+ξ2

Element endpoint displacements and rotations

zE,loce=Te·zEe

u1e=take1;zE,loce , v1e=take2;zE,loce , φ1e=take3;zE,loce

u2e=take4;zE,loce , v2e=take5;zE,loce , φ2e=take6;zE,loce

Displacement functions

ue;ξ=u1e·1-ξ+u2e·ξ

ve;ξ=v1e·Φ1e;ξ+φ1e·Φ2e;ξ+v2e·Φ3e;ξ+φ2e·Φ4e;ξ

Deformed shape, mm